01. First, let's install the official Raspberry Pi GPIO library for Python. It makes controlling pins incredibly easy. Run this in your terminal:

sudo apt update

sudo apt install python3-gpiozero -y

02. Step 1: Tell the Firmware to Blink the LED

The Raspberry Pi has a built-in feature to map its internal "activity" LED (which naturally blinks while the Pi is reading from the boot drive) to an external GPIO pin.

2.1 Open the Pi's low-level hardware configuration file:

sudo nano /boot/firmware/config.txt

2.2 Scroll to the very bottom of the file and add this exact line:

dtoverlay=act-led,gpio=27

03. Now, let's create the script. We will place it in your user directory.

sudo mkdir /usr/local/bin/heath

cd /usr/local/bin/heath

sudo nano health_monitor.py

04. Paste the following Python code into the file. I have designed the logic exactly as you requested: It assumes the Pi is "booting" for the first 60 seconds (Green blinks), then goes solid Green. It checks the temp every 5 seconds, and if it exceeds 70°C, the Red LED blinks rapidly.

from gpiozero import LED, Buzzer

import time

import os

from datetime import datetime

# Setup Pins

disk_led = LED(17) # Disk Activity

buzzer = Buzzer(16) # Temperature Warning

# Configuration

TEMP_THRESHOLD = 50.0

SCRIPT_DIR = os.path.dirname(os.path.abspath(__file__))

LOG_FILE = os.path.join(SCRIPT_DIR, "nas_status.log")

def get_uptime():

with open('/proc/uptime', 'r') as f:

return float(f.readline().split()[0])

def get_temp():

with open('/sys/class/thermal/thermal_zone0/temp', 'r') as f:

return float(f.read()) / 1000.0

def get_disk_activity():

"""Reads total sectors read and written across all drives."""

activity = 0

with open('/proc/diskstats', 'r') as f:

for line in f:

parts = line.split()

if len(parts) >= 10:

dev = parts[2]

# Tracks SATA (sd), RAID (md), NVMe (nvme), and SD Card (mmcblk)

if dev.startswith(('sd', 'md', 'nvme', 'mmcblk')):

activity += int(parts[5]) # Sectors Read

activity += int(parts[9]) # Sectors Written

return activity

# Initial State Setup

last_disk_activity = get_disk_activity()

last_log_time = time.time()

buzzer_is_beeping = False

print("NAS Monitor Started: Green=Power, Red=Disk I/O, Buzzer=Temp...")

try:

while True:

current_time = time.time()

# --- 1. DISK ACTIVITY LOGIC (Disk LED) ---

current_disk_activity = get_disk_activity()

# If read/write sectors increased, turn Disk LED ON momentarily

if current_disk_activity > last_disk_activity:

disk_led.on()

else:

disk_led.off()

last_disk_activity = current_disk_activity

# --- 2. TEMPERATURE LOGIC & BUZZER (Runs every 5 seconds) ---

if current_time - last_log_time >= 5.0:

temp = get_temp()

uptime = get_uptime()

current_dt = datetime.now().strftime("%Y-%m-%d %H:%M:%S")

# Write to log

with open(LOG_FILE, 'a') as log:

log.write(f"[{current_dt}] Uptime: {uptime:.2f}s | Temp: {temp:.2f}°C\n")

# Update Buzzer status

if temp >= TEMP_THRESHOLD and not buzzer_is_beeping:

buzzer.blink(on_time=0.5, off_time=0.5) # Urgent Beep

buzzer_is_beeping = True

elif temp < TEMP_THRESHOLD and buzzer_is_beeping:

buzzer.off()

buzzer_is_beeping = False

last_log_time = current_time

# Check disk activity extremely fast for a responsive flicker

time.sleep(0.05)

except KeyboardInterrupt:

pass

finally:

# Clean up on exit

disk_led.off()

buzzer.off()

05. Automate it on Startup

5.1 Create the service file:

sudo nano /etc/systemd/system/nasleds.service

5.2 Paste this configuration in (assuming your Ubuntu username is ubuntu. If you made a custom username when flashing the OS, change User=ubuntu and the ExecStart path to match your username!):

[Unit]

Description=NAS Heath Status Monitor

After=multi-user.target

[Service]

Type=simple

User=root

ExecStart=/usr/bin/python3 /usr/local/bin/heath/health_monitor.py

Restart=always

RestartSec=5

[Install]

WantedBy=multi-user.target

5.3Tell Ubuntu to recognize the new service and start it immediately:

sudo systemctl daemon-reload

sudo systemctl enable nashealth.service

sudo systemctl start nashealth.service

5.4 For Edits

sudo systemctl daemon-reload